bladeRF — Hardware Components & Technical Specifications

There are two major bladeRF hardware generations. I'll separate them because their RF chips and FPGA are different:

- bladeRF 1.x / Classic: x40 and x115

- bladeRF 2.0 micro: xA4, xA5, xA9

1. bladeRF 2.0 micro

6

Nuand describes the bladeRF 2.0 micro as a 2×2 MIMO USB 3.0 SDR covering 47 MHz–6 GHz, with up to 61.44 MSPS sampling and 56 MHz filtered bandwidth.

Main technical specifications

| Parameter | bladeRF 2.0 micro |
| --- | --- |
| Architecture | 2×2 MIMO |
| RX channels | 2 |
| TX channels | 2 |
| RX tuning range | 70 MHz–6 GHz |
| TX tuning range | 47 MHz–6 GHz |
| ADC/DAC resolution | 12-bit |
| Sample-rate range | 0.521–61.44 MSPS |
| Maximum filtered bandwidth | 56 MHz |
| Typical CW output | +8 dBm |
| USB | USB 3.0 SuperSpeed |
| FPGA | Intel/Altera Cyclone V E |
| FPGA variants | xA4, xA5, xA9 |
| Clock | 38.4 MHz VCTCXO |
| External reference | Yes |
| Operating temperature | 0–70 °C |
| Dimensions | 6.3 × 10.2 × 1.8 cm |
| Weight | ~90 g |

2. bladeRF 2.0 micro hardware architecture

The major signal path is:

RX1 SMA ─────┐

│

RX2 SMA ─────┤

▼

┌───────────┐

│           │

│  AD9361   │

│ RF        │

│ Transceiver

│           │

└─────┬─────┘

│

I/Q data

│

▼

┌─────────────┐

│ Cyclone V E │

│    FPGA     │

└──────┬──────┘

│

GPIF II

│

▼

┌─────────────┐

│ Cypress FX3│

│ USB 3.0    │

└──────┬──────┘

│

USB 3.0

│

▼

PC

The FPGA interfaces directly with the AD9361 RF transceiver and Cypress FX3 USB controller, while also interfacing with the clock system, VCTCXO trim circuitry, GPS/1-PPS and expansion header.

3. AD9361 RF transceiver

The Analog Devices AD9361 is the central RF component.

It provides:

- 2 RX channels

- 2 TX channels

- RF frequency synthesis

- RX/TX mixers

- programmable gain

- analog filtering

- ADCs

- DACs

- baseband interfaces

- I/Q data interfaces

Therefore:

Antenna

│

▼

RF front-end

│

▼

AD9361

┌──────┴──────┐

│             │

RX1           RX2

│             │

TX1           TX2

│             │

└──────┬──────┘

│

I/Q

│

▼

FPGA

Nuand specifically identifies the AD9361 as the integrated RF transceiver used in the bladeRF 2.0 micro.

4. Cyclone V FPGA

The second major component is the Intel/Altera Cyclone V E FPGA.

The bladeRF 2.0 micro is available with different FPGA capacities:

| Model | FPGA capacity |
| --- | --- |
| xA4 | 49 kLE |
| xA5 | Intermediate capacity |
| xA9 | 301 kLE |

The xA9 provides substantially more FPGA resources for custom DSP, FFTs, filters, demodulators, decoders and other hardware processing.

FPGA resources

For the published xA4–xA9 range:

- Logic elements: 49–301 kLE

- Embedded memory: 3,383–13,917 kbits

- Variable-precision DSP blocks: 66–342

- 18×18 multipliers: 132–684

5. Cypress FX3 USB controller

The bladeRF uses the Cypress FX3 USB 3.0 controller.

Its job is primarily to transfer high-speed I/Q data:

AD9361

↓

Cyclone V FPGA

↓

FX3

↓

USB 3.0

↓

Computer

The FX3 provides the USB 3.0 SuperSpeed connection to the host computer. Nuand's hardware description identifies the FX3 as one of the main devices in the architecture.

6. Clock system

The board uses a 38.4 MHz factory-calibrated VCTCXO.

Nuand specifies:

- 38.4 MHz reference

- factory calibration

- frequency taming capability

- 12-bit DAC control

- optional ADF4002 PLL-based taming

- synchronization/reference interfaces

The FPGA also interfaces with the clock generator and VCTCXO trim circuitry.

7. Flash memory

The SPI flash stores important device information.

For the bladeRF 2.0 micro:

| Model | Flash | Voltage |
| --- | --- | --- |
| xA4 | W25Q32JVSSIQ | 3.3 V |
| xA9 | W25Q128JVSSIQ | 3.3 V |

The flash can contain:

- FX3 firmware

- calibration data

- FPGA autoload metadata

- FPGA bitstream

- device information

Nuand documents the flash layout and individual flash part numbers.

8. Nios II processor

The FPGA can contain an Intel Nios II soft processor.

It is used for board control and management rather than replacing the host computer.

The FPGA architecture handles:

- RFIC control

- clock control

- GPIO

- I/Q sample handling

- USB data path

- peripheral management

9. RF connectors

The bladeRF 2.0 micro provides:

RX1 ───── SMA

RX2 ───── SMA

TX1 ───── SMA

TX2 ───── SMA

This gives the board its 2×2 MIMO architecture.

10. Expansion and synchronization hardware

The FPGA also connects to the expansion interface.

The hardware architecture provides interfaces for:

- GPIO

- external control

- GPS-related NMEA input

- 1-PPS synchronization

- reference clock

- expansion boards

Nuand's HDL documentation explicitly identifies the GPS NMEA, 1-PPS, reference-clock and expansion-header connections.

11. Power system

The bladeRF 2.0 micro can operate from:

- USB 3.0 power

- external 5-V DC input

The board includes power-management circuitry that automatically selects between USB bus power and external DC power.

12. Complete major-component list

bladeRF 2.0 micro

| Section | Component |
| --- | --- |
| RF transceiver | Analog Devices AD9361 |
| FPGA | Intel/Altera Cyclone V E |
| USB controller | Cypress EZ-USB FX3 |
| Reference oscillator | 38.4 MHz VCTCXO |
| VCTCXO control | 12-bit DAC / taming circuitry |
| Optional clock taming | ADF4002 PLL |
| Flash xA4 | Winbond W25Q32JVSSIQ |
| Flash xA9 | Winbond W25Q128JVSSIQ |
| FPGA CPU | Nios II soft processor |
| RF connectors | SMA |
| RF switching | RF switch network |
| RF filtering | RF filters/matching networks |
| Power | USB + 5-V DC power circuitry |
| Synchronization | Reference / 1-PPS |
| Expansion | GPIO/expansion header |

13. bladeRF 1.x / Classic

The original bladeRF is substantially different.

Technical specifications

| Parameter | bladeRF Classic |
| --- | --- |
| RF range | 300 MHz–3.8 GHz |
| RX | 12-bit |
| TX | 12-bit |
| Sample rate | Up to 40 MSPS |
| Full-duplex bandwidth | Up to 28 MHz |
| RF transceiver | LMS6002D |
| FPGA | Cyclone IV E |
| FPGA variants | x40 / x115 |
| USB | USB 3.0 SuperSpeed |
| Clock | 38.4 MHz VCTCXO |
| Clock generator | Si5338 |
| USB controller | Cypress FX3 |
| CPU | ARM9 inside FX3 |
| MIMO | 2×2 possible |
| Expansion | GPIO/expansion connector |

14. bladeRF Classic components

RX/TX SMA

│

▼

RF switching

│

▼

┌────────────┐

│ LMS6002D   │

│ RFIC       │

└─────┬──────┘

I/Q

│

▼

┌────────────┐

│ Cyclone IV │

│ E FPGA     │

└─────┬──────┘

│

GPIF II

│

▼

┌────────────┐

│ Cypress    │

│ FX3        │

└─────┬──────┘

│

USB 3.0

The Classic FPGA also controls the Si5338 clock generator, VCTCXO DAC, RF switches and expansion interface.

15. x40 vs x115

The two Classic models primarily differ in FPGA capacity:

| Model | FPGA |
| --- | --- |
| bladeRF x40 | Cyclone IV E, ~40K logic elements |
| bladeRF x115 | Cyclone IV E, ~115K logic elements |

Nuand notes that FPGA size is the key difference between x40 and x115 within the same generation.

16. BladeRF generation comparison

| Feature | bladeRF x40/x115 | bladeRF 2.0 micro |
| --- | --- | --- |
| RF transceiver | LMS6002D | AD9361 |
| Frequency | 300 MHz–3.8 GHz | 47 MHz–6 GHz TX / 70 MHz–6 GHz RX |
| ADC/DAC | 12-bit | 12-bit |
| Max sample rate | 40 MSPS | 61.44 MSPS |
| Bandwidth | 28 MHz | 56 MHz |
| MIMO | 2×2 | 2×2 |
| FPGA | Cyclone IV E | Cyclone V E |
| USB | USB 3.0 | USB 3.0 |
| Clock | 38.4 MHz | 38.4 MHz |
| Main RF architecture | LMS6002D | AD9361 |

Most important documents for component-level study

For a serious bladeRF hardware technical reference, I would collect these in this order:

- bladeRF 2.0 micro schematic

- bladeRF 2.0 micro BOM

- AD9361 datasheet

- Cyclone V E datasheet

- Cypress FX3 datasheet

- VCTCXO datasheet

- Clock/PLL documentation

- SPI flash datasheet

- bladeRF FPGA HDL

- FX3 firmware source

- libbladeRF documentation

- bladeRF hardware revision documentation

Nuand makes the schematics, HDL, firmware and software available as open-source material, making bladeRF one of the better SDRs for component-level hardware study.